# ESP32-CAM + LAPTOP AUDIO MODEL – COMPLETE PROJECT DOCUMENT

# 1. OVERVIEW

This document covers the setup where the ESP32-CAM captures images and the laptop itself speaks the output using laptopserver.py.

✔ Laptop converts text → audio and plays it directly.
✔ No speaker amplifier or ESP32-S3 used.
✔ ESP32‑CAM + Button + Ultrasonic alert system.

# 2. ESP32‑CAM SYSTEM (freshhhcam.ino)

- Blind person presses button.

- ESP32‑CAM captures high‑resolution picture.

- Encodes to Base64.

- Sends to laptopserver.py.

- Server processes and plays back through laptop speakers.

### Button Wiring

| Button | ESP32‑CAM |
| --- | --- |
| GND | GND |
| VCC | 3.3V |
| OUT | IO13 |

### Flash LED Wiring

| LED | ESP32‑CAM |
| --- | --- |
| + | IO4 |
| – | GND |

# 3. ESP32‑CAM Programming Setup

### UPLOAD MODE (Important!)

Connect Arduino Uno → ESP32-CAM:

| Arduino | ESP32‑CAM |
| --- | --- |
| 5V | 5V |
| GND | GND |
| TX | U0R |
| RX | U0T |
| RESET → GND | (only during upload) |
| (only during upload) | IO0 → GND |

After upload: - REMOVE IO0→GND - REMOVE RESET→GND - Leave TX & RX connected - Power CAM using Arduino’s 5V

# 4. ESP32 BUZZER + ULTRASONIC SYSTEM (buzzer.ino)

A second ESP32 continuously detects obstacles.

### Ultrasonic → ESP32

| HC‑SR04 | ESP32 |
| --- | --- |
| VCC | VIN |
| GND | GND |
| TRIG | D5 |
| ECHO | D18 |

### Buzzer → ESP32

| Buzzer | ESP32 |
| --- | --- |
| + | D15 |
| – | GND |

If obstacle < 100 cm → buzzer rings for 2 seconds. Blind user then presses camera button.

# 5. PYTHON SERVER (laptopserver.py – Laptop Audio)

### This server:

✔ Receives Base64 image from ESP32‑CAM
✔ Sends to OpenRouter Vision Model
✔ Extracts readable text + objects
✔ Cleans the text
✔ Converts to MP3 (gTTS)
✔ Converts MP3 → WAV using FFmpeg
✔ Plays WAV sound DIRECTLY on laptop speakers using pygame

### HOW TO RUN

py -3.10 laptopserver.py

Laptop begins speaking outputs.

# 6. FULL WORKFLOW

- Ultrasonic sensor detects obstacle.

- Buzzer beeps → blind user presses button.

- ESP32‑CAM captures image.

- Image sent to laptop server (laptopserver).

- Laptop: Vision analysis → OCR → Object detection.

- Laptop generates speech.

- Laptop plays audio automatically.

# 7. FILES USED

- freshhhcam.ino → ESP32-CAM

- buzzer.ino → Ultrasonic + buzzer ESP32

- laptopserver.py → Laptop audio server

# 8. COMMAND SUMMARY

| Purpose | Command |
| --- | --- |
| Run laptop audio server | py -3.10 laptopserver.py |
| Upload camera code | Arduino IDE (IO0→GND) |
| Run ultrasonic buzzer | Upload buzzer.ino to 2nd ESP32 |

This document is for the Laptop Audio Model (laptopserver) where all sound comes directly from the computer.